| VARIA CONNECT · PERMITS TO WORK Confined Space Entry Permit Permit Kemasukan Ruang Terkurung | Form PTW-CSE Company logo |
| --- | --- |

When to use: Entry into manholes, sumps, tanks, pits, ducts, culverts or any space with limited entry and exit and possible poor air. Follows the DOSH Industry Code of Practice for Safe Working in a Confined Space 2010.

Validity / frequency: One shift; retest gas after every break

PERMIT DETAILS

| Project / site |  |
| --- | --- |
| Permit no. |  |
| Date |  |
| Exact location (block / level / grid) |  |
| Description of work |  |
| Company / subcontractor |  |
| Permit holder (in charge on site) |  |
| Number of workers |  |
| Valid from |  |
| Valid until |  |
| Confined space name / ID |  |
| Authorised Entry Supervisor (AES) |  |
| Authorised Gas Tester (AGT) |  |
| Standby person (AESP) at entrance |  |

PRECAUTIONS BEFORE ENTRY

| No. | Item | Yes | No | N/A | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1 | All entrants and standby person trained as AESP (valid certificate) | ☐ | ☐ | ☐ |  |
| 2 | Space isolated: pipes blanked, valves locked, power locked out (LOTO) | ☐ | ☐ | ☐ |  |
| 3 | Space ventilated; forced ventilation running throughout the work | ☐ | ☐ | ☐ |  |
| 4 | Gas test done from the top, middle and bottom of the space | ☐ | ☐ | ☐ |  |
| 5 | Continuous personal gas detector worn by each entrant | ☐ | ☐ | ☐ |  |
| 6 | Rescue tripod, winch and harness set up at the entrance | ☐ | ☐ | ☐ |  |
| 7 | Standby person stays at the entrance at all times and never enters | ☐ | ☐ | ☐ |  |
| 8 | Communication method agreed (radio / signal rope) | ☐ | ☐ | ☐ |  |
| 9 | Entry log kept at the entrance | ☐ | ☐ | ☐ |  |
| 10 | Emergency services number and nearest hospital noted | ☐ | ☐ | ☐ |  |

ATMOSPHERIC TEST RECORD

| Time | O₂ % (19.5–23.5) | LEL % (<10) | H₂S ppm (<10) | CO ppm (<25) | Tested by (AGT) |
| --- | --- | --- | --- | --- | --- |

ENTRY LOG

| Name | Time in | Time out | Signature |
| --- | --- | --- | --- |

PPE REQUIRED

| ☐ Safety helmet with chin strap | ☐ Safety boots |
| --- | --- |
| ☐ High-visibility vest | ☐ Safety glasses / goggles |
| ☐ Gloves (specify type) | ☐ Full-body harness with lifeline |
| ☐ Breathing apparatus (if required) | ☐ Personal gas detector |

AUTHORISATION

| Role | Name | Signature | Date & time |
| --- | --- | --- | --- |
| Applicant / permit holder: I understand the hazards and will follow every precaution listed |  |  |  |
| Safety & Health Officer / Site Safety Supervisor: site checked, precautions in place |  |  |  |
| Site manager (issuer): permit approved |  |  |  |

HAND-BACK AND CLOSURE

| No. | Item | Yes | No | N/A | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1 | Work completed / stopped (circle which) | ☐ | ☐ | ☐ |  |
| 2 | All workers and tools withdrawn from the area | ☐ | ☐ | ☐ |  |
| 3 | Area left clean and safe; barriers removed or left in place as agreed | ☐ | ☐ | ☐ |  |
| 4 | Isolations removed or handed back (if any) | ☐ | ☐ | ☐ |  |

CLOSURE SIGN-OFF

| Role | Name | Signature | Date & time |
| --- | --- | --- | --- |
| Permit holder |  |  |  |
| Issuer / SHO |  |  |  |

Keep the original at the site office for the duration of the project. Display permits at the work area while work is in progress.