| VARIA CONNECT · PERMITS TO WORK Hot Work Permit Permit Kerja Panas | Form PTW-HOT Company logo |
| --- | --- |

When to use: Welding, gas cutting, grinding, brazing, torch-on waterproofing and any work producing sparks or open flame.

Validity / frequency: One shift; fire watch continues 30 minutes after work ends

PERMIT DETAILS

| Project / site |  |
| --- | --- |
| Permit no. |  |
| Date |  |
| Exact location (block / level / grid) |  |
| Description of work |  |
| Company / subcontractor |  |
| Permit holder (in charge on site) |  |
| Number of workers |  |
| Valid from |  |
| Valid until |  |
| Type of hot work | ☐ Arc welding ☐ Gas cutting / welding ☐ Grinding ☐ Torch-on membrane ☐ Other |

PRECAUTIONS BEFORE WORK STARTS

| No. | Item | Yes | No | N/A | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1 | Combustible materials removed within 11 m, or covered with fire blankets | ☐ | ☐ | ☐ |  |
| 2 | Floor and wall openings covered to stop sparks falling to levels below | ☐ | ☐ | ☐ |  |
| 3 | Fire extinguisher (min. 9 kg dry powder) at the work point | ☐ | ☐ | ☐ |  |
| 4 | Trained fire watcher assigned, with no other duties | ☐ | ☐ | ☐ |  |
| 5 | Gas cylinders upright, chained, with flashback arrestors and regulators checked | ☐ | ☐ | ☐ |  |
| 6 | Welding set earthed; cables free of damage | ☐ | ☐ | ☐ |  |
| 7 | Gas test done if work is near drains, tanks or enclosed spaces | ☐ | ☐ | ☐ |  |
| 8 | Fire alarm / smoke detectors in the area isolated (if any) and fire team informed | ☐ | ☐ | ☐ |  |

GAS TEST (IF REQUIRED)

| Time | LEL % (must be 0) | O₂ % (19.5–23.5) | Tested by |
| --- | --- | --- | --- |

PPE REQUIRED

| ☐ Safety helmet with chin strap | ☐ Safety boots |
| --- | --- |
| ☐ High-visibility vest | ☐ Safety glasses / goggles |
| ☐ Gloves (specify type) | ☐ Welding helmet / face shield |
| ☐ Leather welding gloves & apron | ☐ Fire-resistant clothing |

AUTHORISATION

| Role | Name | Signature | Date & time |
| --- | --- | --- | --- |
| Applicant / permit holder: I understand the hazards and will follow every precaution listed |  |  |  |
| Safety & Health Officer / Site Safety Supervisor: site checked, precautions in place |  |  |  |
| Site manager (issuer): permit approved |  |  |  |

FIRE WATCH AFTER WORK

| Hot work finished at |  |
| --- | --- |
| Fire watch ended at (min. 30 min later) |  |
| Fire watcher name |  |

HAND-BACK AND CLOSURE

| No. | Item | Yes | No | N/A | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1 | Work completed / stopped (circle which) | ☐ | ☐ | ☐ |  |
| 2 | All workers and tools withdrawn from the area | ☐ | ☐ | ☐ |  |
| 3 | Area left clean and safe; barriers removed or left in place as agreed | ☐ | ☐ | ☐ |  |
| 4 | Isolations removed or handed back (if any) | ☐ | ☐ | ☐ |  |

CLOSURE SIGN-OFF

| Role | Name | Signature | Date & time |
| --- | --- | --- | --- |
| Permit holder |  |  |  |
| Issuer / SHO |  |  |  |

Keep the original at the site office for the duration of the project. Display permits at the work area while work is in progress.